[image: image1.png]0 N
ermat

ro

L CormatProltd 4



CERMATPRO LTD

PRODUCT HEALTH AND SAFETY DATA

	DATA SHEET NO.
	JP/HSO/01

	DATE ISSUED
	15/12/2015

	NO.OF PAGES
	4


	1. PRODUCT & COMPANY IDENTIFICATION


	PRODUCT NAME
	JPVest 

	ADDITIONAL INFO
	www.cermatpro.com

	CAS No
	1309-48-4, 7722-76-1, 60676-86-0             

	EINECS No
	215-171-9, 231-764-5, 262-373-8


Contact Manufacturer
Cermatpro Ltd

Granimar House

Cooting Road

Aylesham

Kent

CT3 3EP

UK
	2. HAZARDS IDENTIFICATION


	Emergency Overview
	Non-combustible

	Eye Contact
	Normal problems caused by inert particulate matter

	Skin Contact
	May cause irritation

	Inhalation
	No known effects other than respiratory irritation

	Ingestion
	None known

	Classification
	Not dangerous

	Aggravated medical conditions
	Asthma, lung irritation, dermatitis


	3. COMPOSITION / INFORMATION OF INGREDIENTS


	CHEMICAL NAME
	Mono ammonium phosphate

	COMPOSITION
	M.E.L.

(mg per cbm)
	O.E.S.

(mg per cbm)

	Stable mixture of mainly silica(SiO2) with minor amounts of P2O5, MgO, TiO2,Fe2O3, Na2O, etc.
	N/A

(See below)
	N/A

(See below)


	MINERALOGY
	M.E.L.

(mg per cbm)
	O.E.S.

(mg per cbm)

	Major component – silica

Minor component – phosphate

Minor component - periclase
	N/A

(See below)
	N/A

(See below)


	4. FIRST AID MEASURES


	Primary routes of body entry
	Over-Exposure, Acute & Chronic Effects
	First Aid & Medical Information

	Inhalation
	No known effects other than respiratory irritation
	Remove from exposure

	Ingestion
	None known.
	None applicable

	Skin contact
	None known, fine powders may dry the skin. In some cases sharp grains could abrade the skin.
	General first aid procedures.

	Skin absorption
	Not applicable.
	Not applicable.

	Eye
	Normal problems caused by inert particulate matter.
	Do not rub, irrigate well, seek medical advice if matter not removed.

	Other potential health risks, e.g. Synergistic Reactions
	None known.
	Not applicable.

	Toxicity
	None known to date but low to non-existent.
	Not applicable.


	5. FIRE-FIGHTING MEASURES


	Flash point
	N/A
	Extinguishing

media
	Not a fire hazard

	Special fire fighting procedure
	Has fire retardant properties

                                                    Governed by other materials present

                                                    No special fire-fighting equipment or measures required
	Explosion 

potential
	None. Normal very fine dust explosion risk - no recorded occurrence.


	6. ACCIDENTAL RELEASE MEASURES


	Personal precautions
	Wear personal protective equipment

Avoid dust formation

	Environmental precautions
	No special environmental precautions required

	Methods for clean-up
	Place in a fibre keg or paper sack and dispose as                                                    Industrial waste.


	7. HANDLING & STORAGE


	Normal packing method
	Heat sealed packets

	Normal use
	Normal dental applications

	Procedures in event of spillage
	Retrieval by means of vacuum is recommended to avoid creation of nuisance dust.

	Waste disposal
	Powder may be dumped as industrial waste

	Special handling precautions
	None.


	8. EXPOSURE CONTROLS / PERSONAL PROTECTION


This material has not been assigned specified Occupational Exposure (O.E.L.) under COSHH and with no indication of a need for a lower value, personal exposure should be kept below 10mg per cbm 8 hour T.W.A. for “total inhalable dust” 1or below 5 mg per cbm 8 hour T.W.A. for “respirable dust”1. Such or larger dust concentrations would be taken as “substantial concentrations” and such are substances hazardous to health.

T.W.A. = Time Weighted Average. 

1 See MDHS 14: General methods for the gravimetric determination of respirable and total inhalable dust. ISBN 0 7176 03431

	PERSONAL PROTECTION REQUIREMENTS

	Respiratory protection
	Personal dust masks suitable for nuisance dust.

	Ventilation
	As dictated by process to minimise nuisance dust.

	Protective gloves
	Advisable in cases where skin drying could occur.

	Eye protection
	Advisable to prevent ingress of dust into eyes.

	Other equipment
	Advisable to wear safety footwear when handling palletised loads.


PRECAUTIONS TO BE TAKEN DURING REPAIR AND MAINTENANCE OF CONTAMINATED EQUIPMENT WITH WHICH THIS MATERIAL HAS BEEN IN CONTACT:
Normal hygiene standards and use of personal safety equipment such as dust masks and eye protection.

	9. PHYSICAL & CHEMICAL PROPERTIES


	PHYSICAL DATA

	Boiling Point
	N/A
	Solubility-water
	One component is soluble in water.

	Melting Point
	One component with low melting point. When set melting point in excess of 2000oC
	Solubility-alcohol
	One component is soluble in alcohol.

	Vapour pressure
	N/A
	Solubility-others
	Minor in Hydrofluoric acid

	Vapour density
	N/A
	Appearance
	Off-white/grey

	Specific gravity
	1.00
	Odour
	None


	10. STABILITY & REACTIVITY


	Stability
	Stable at low temperatures

	Polymerisation
	Unknown

	Incompatible with
	None known

	Decomposition Products
	None

	Conditions to be avoided
	Strong alkalis will liberate ammonia


	11. TOXICOLOGICAL INFORMATION


Local Effects
	Skin irritation
	May cause skin irritation

	Eye irritation
	May cause eye irritation

	Inhalation
	May cause respiratory tract irritation

	Ingestion
	None known


Long Term Effects

	Skin irritation
	None known

	Eye irritation
	None known

	Inhalation
	None known

	Ingestion
	None known


	12. ECOLOGICAL INFORMATION


	Additional ecological information
	No information available


	13. DISPOSAL CONSIDERATIONS


	Waste disposal method
	Dispose of in accordance with local regulations

	Contaminated packaging
	Empty containers should be taken for local recycling, recovery or waste disposal

	Further information
	Can be land-filled, when in compliance with local regulations


	14. TRANSPORT INFORMATION


Not classified as hazardous for transport.
	15. REGULATORY INFORMATION


Not classified as a substance hazardous for supply.
	16. OTHER INFORMATION


OTHER INFORMATION - None available.

NOTE:
The above information is believed to be correct at the date of publication. Any further information on the safety aspects of our products will be supplied as and when this becomes available.

CERMATPRO LIMITED
GRANIMAR HOUSE,

COOTING ROAD,

AYLESHAM,

KENT CT3 3EP.

UNITED KINGDOM.

TELEPHONE: +44 1304-842222

FAX: +44 1304-842434

E-mail: info@cermatpro.com
PAGE  
4

